*NOTE: Your files may be automated to display in AutoCad from the source CD without the following instructions. If you receive a vector file and an image file at different times or if you wish to reference the ortho image to data files not referenced to that image or located in alternative source directories, please follow these instructions.

To Attach Ortho(s) To An AutoCad Drawing

Make certain that ALL files are in the same source directory!

1.	Determine the coordinates of the lower left coordinates of the ortho(s).

These coordinates can be found on the list of MBR coordinates provided in an ASCII txt file provided with your delivery. This file differs from the TFW file also provided as a correlation source file for image georeferencing.

2.	Determine the scale factor for each ortho to be inserted.

This scale factor is the width of the ortho in whatever units the ortho was created (feet/meters).

This can be determined by subtracting the lower left X coordinate from the upper right X coordinate.

3.	Open AutoCad.

4.	Open the appropriate AutoCad drawing.

5.	Under Insert choose Raster Image.

6.	Click on Attach, then Browse and select the ortho to be opened and click              Open.

7.	Uncheck any box that says Specify on-screen.

8.	In the box marked At: enter the lower left X and Y coordinates.

9.	In the box marked Scale factor: enter the predetermined scale factor for the ortho selected.

10.	Box marked Rotation angle should remain at 0.

11.	Click on OK.

12.	Regenerate the drawing to display the vector data on top on the ortho(s).

13.	Save the drawing with the ortho(s) attached. Again make sure ALL files are in the same directory!

AutoCad Map Users:

- Under Map, choose Image, choose Insert, choose Browse file.TIFF – Insert Box should be set to World File under the Correlation Source option. Choose OK. Zoom extents to regenerate and display the file.

- If displaying an image file with a vector file, go to the Tools option, choose Display Order, choose Send To Back, then manually use your cursor and pick the exterior limits of the image. Your image should now regenerate with the vector data as the foreground and the image as a background.